TF N 530 Rev 2

2012-04-13

CONSTRUCTION SECTOR STANDARDIZATION

GUIDANCE DOCUMENT

Implementation of the Construction Products Regulation (CPR) in harmonized standards

- Template for Annex ZA -

Drafting the Annex ZA

Foreword

The Annex ZA is the part of the standard which is used for the purpose of CE marking of construction products and transforms all or part of a European product standard into a harmonized European product standard.

The Annex ZA identifies those clauses of the standard (or refers to clauses in another standard) which cover the essential characteristics included in the mandate to which it responds, considered in the answer to the mandates prepared by the TC and accepted by the Commission.

Moreover, it reproduces the Assessment and Verification of Constancy of Performance (AVCP) system(s) of product(s) established by Commission acts, to which the concerned product(s) needs to be submitted before the manufacturer is entitle to draw up the Declaration of Performance (DoP) and to affix the CE marking.

Introduction

(1)	This document has been prepared taking into account the experience gained in the drafting of Annex ZA under Council Directive 89/106/EEC and the adoption of the Regulation (EU) No. 305/2011(CPR).

(2)	Clause 1 of this document provides the general guidance including explanations regarding the meaning of the different parts of clause 2.

(3)	Clause 2 of this document provides the template that new, revised or amended (candidate) harmonized standards shall contain when submitted to FV or UAP (Stage 50.20.0000), after 1st July 2013.

(4) In the template given in clause 2 of this document:

-	the text in [blue character] represents items to be filled in;

- the text in [black italics] represents notes or other comments not to be included in the standard.

1	General guidance

## Meaning of the Annex ZA

### General

(1) 	The Annex ZA is the part of the standard which, without prejudice to the simplified measures envisaged by the CPR, needs to be applied for CE marking purposes and transforms all or part of a European product standard into a harmonized European product standard.

(2) 	Only (candidate) harmonized product standards include Annex ZA; supporting standards do not.

(3) 	It identifies those clauses of the standard (or refers to clauses in another standard) which cover the essential characteristics included in the mandate to which it responds and considered in the answer to it, as prepared by the TC and accepted by the Commission.

(4)	Moreover, it reproduces the system(s) for Assessment and Verification of Constancy of Performance (AVCP) of product(s) established by EC Decisions, to which the concerned product needs to be submitted before the manufacturer is entitled to draw up the DoP and to affix the CE marking.

(5) 	Annex ZA, although it is an informative annex, contains information on those parts of the standard which relate to regulatory requirements and has effects for CE marking.

### Clause ZA.1 - Scope and relevant characteristics

(1) 	This clause of Annex ZA indicates, in its Tables ZA.1.1 to ZA.1.n, its scope, and shall refer to the Scope (Clause 1) of the standard. Usually the coverage of Annex ZA will be the same as the scope of the standard itself, although in some cases it may be more limited.

(2) 	It is not possible, however, for Annex ZA to have a wider coverage than the scope of the standard itself.

(3) 	Tables ZA.1.1 to ZA.1.n shall list all essential characteristics (including the assessment of durability aspects) as identified in the mandate, considered as relevant in the TC’s answer to the mandate and accepted by the European Commission as being appropriate, and the clause(s) of this (or another) standard which cover each of them.

(4) 	Clause 4 in the body of the standard shall be sufficiently well differentiated so that reference to a clause does not bring other requirements, not covered by the mandate, into the harmonized part of the EN.

(5) 	In general, one clause should refer to one characteristic only. The table needs only referring to clause 4, not test method(s) (although these may be given in the column “Notes” in Tables ZA.1.1 to ZA.1.n), because the test/assessment/calculation method to be used should be uniquely identifiable from the relevant clauses.

(6) 	The third column in Tables ZA.1.1 to ZA.1.n, titled "Regulatory classes", is limited to regulatory levels and classes of performance.

(7) 	The last column in Tables Z.1.1 to ZA.1.n titled "Notes", should be used to indicate how results are to be expressed (e.g. the units, values, etc.), threshold values or (technical) classes of performance.

(8) 	The clauses to which Tables ZA.1.1 to ZA.1.n refer should allow the test/assessment/calculation method used to be unambiguously identified. This will usually be done by the requirement text in Clause 4 of the standard referring to a  test/assessment/calculation method in Clause 5 “Testing, assessment and sampling methods” or to another standard (see the model standard).

(9) 	Tables ZA.1.1 to ZA.1.n may, however, refer to clauses in another standard, if necessary.

(10) 	If an EN covers only mandated essential characteristics, Tables ZA.1.1 to ZA.1.n will then refer to the whole essential characteristics in clause 4 of the standard

(11) 	More often, however, it will refer to only some of the clauses, where the standard includes other non-harmonized requirements.

(12) 	Where a product covered by the standard has distinctly different end uses (i.e. having different characteristics), or where it covers products subject to more than one mandate, Tables ZA.1.1 to ZA.1.n shall be subdivided accordingly.

##### 1.1.3	Clause ZA.2 - Procedure(s) for the Assessment and Verification of Constancy of Performance (AVCP)

#### 1.1.3.1	ZA.2.1 AVCP system(s)

(1) 	Annex ZA shall contain a sub-clause, ZA.2.1, which identifies the relevant AVCP system(s) and the tasks of the different parties, i.e. who will do the assessment and the verification.

(2) 	Only Annex ZA can contain details about the assignment of third party tasks.

(3) 	ZA.2.1 shall reproduce the relevant AVCP system(s) as any other prescriptions established by the Commission given in the text of the EC Decision and in the mandate, such as “Regarding products under systems 1 and 3, for the determination of the product-type the tasks for the notified body shall be limited to the following characteristics:…”.

(4) 	When the AVCP system established by the Commission by means of Decisions or delegated acts covers several products and/or several intended uses, those covered by the standard can be extracted together with the relevant text or footnotes, but without changing the wording used. This must be done carefully to ensure that the standard does not inadvertently change the AVCP system or its details. However, where the reference to products included in the above-mentioned acts is too generic, TCs may complement the wording used for making specific reference to product(s). If this is the case, it shall be done outside the Table extracted from the Commission acts.

(5) 	The responsibility for different tasks related to the AVCP shall be presented as in Tables ZA.3.1 to ZA.3.5 in clause 2 of this document. This presentation has been agreed by the European Commission because it represents the roles for different parties given in Annex  V of the CPR.

(6) 	The CPR requires “Testing/Further testing of samples taken at the factory in accordance with a prescribed test plan” for AVCP systems 1+, 1, 2+. This is usually assumed to cover FPC testing of finished products.

(7) 	Clause ZA.2.1 shall also require that the product be subject to the determination of the product-type against the essential characteristics listed in Table ZA.1 for which the manufacturer intends to declare the performance.

(8) 	Annex ZA shall not give requirements on the qualifications or accreditation of notified bodies. This is a legal matter which remains the responsibility of Member States.

(9) In case a construction product is subject to more than one AVCP systems, Annex ZA(s) shall give information on the applicable AVCP  tasks and guidance will be provided on a case by case basis.

#### 1.1.3.2 	Clause ZA.2.2 - Declaration of performance (DoP)

The Annex ZA  contains an example of the possible implementation of the DoP model as provided by Annex III of CPR and reproduced below.  (see clause ZA.2.2.3 “Example of the Declaration of performance”).

Declaration of performance

No. .................. [to be given by the manufacturer]

- Unique identification code of the product-type: [to be given by the manufacturer]

2.	Type, batch or serial number or any other element allowing identification of the construction product as required under Article 11(4): [to be given by the manufacturer in accordance with the provisions included in its FPC ]

3.	Intended use or uses of the construction product, in accordance with the applicable harmonised technical specification, as foreseen by the manufacturer: [to be taken from clause 1 “Scope” of the harmonized standard]

4.	Name, registered trade name or registered trade mark and contact address of the manufacturer as required pursuant Article 11(5): [to be given by the manufacturer and shall correspond to the same information that will be given in the CE marking box]

5.	Where applicable, name and contact address of the authorised representative whose mandate covers the tasks specified in Article 12(2): [to be given by the manufacturer]

6.	System or systems of assessment and verification of constancy of performance of the construction product as set out in Annex V: [to be taken from the last column of Table ZA.2 of Annex ZA of the harmonized standard]

7.	In case of the declaration of performance concerning a construction product covered by a harmonised standard: [the name and identification number of the notified body if the concerned product is covered by AVCP system 1+, 1, 2+ or 3]

Performed: [the description of the tasks performed by the notified body in relation to the applicable AVCP system is to be taken from the relevant paragraph of clause ZA.2.2 of Annex ZA of the harmonized standard, which corresponds to the third party tasks as set out in Annex V of the CPR]

under system [this information is to be taken from the last column of Table ZA.2 of Annex ZA of the harmonized standard]

and issued: [certificate of constancy of performance, certificate of conformity of the factory production control, test/calculation reports - as relevant]

8.	Declared performance

| Essential characteristics (see Note 1) | Performance (see Note 2) | Harmonized technical specification (see Note 3) |
| --- | --- | --- |

Notes to the table:

1.	Column 1 shall contain the list of essential characteristics as determined in the harmonised technical specifications for the intended use or uses indicated in point 3 above [this information is to be taken from Table ZA.1 of Annex ZA of the harmonized standard. The full list of the essential characteristics has to appear maintaining the same sequence and the same wording indicated in Table ZA.1];

2.	For each essential characteristic listed in column 1 and in compliance with the requirements of Article 6 of the CPR, column 2 shall contain the declared performance, expressed by level or class, or in a description, related to the corresponding essential characteristics [this information is to be taken from the last column of Table ZA.1 of Annex ZA of the harmonized standard]. The letters "NPD" (No Performance Determined) shall be indicated where no performance is declared;

3.	For each essential characteristic listed in column 1, column 3 shall contain dated reference of the corresponding harmonised standard and, where relevant, the reference number of the Specific or Appropriate Technical Documentation used;

Where pursuant to Article 37 or 38 the Specific Technical Documentation has been used, the requirements with which the product complies  [where the mentioned simplified procedures are not applicable to the manufacturer responsible for the DoP or where the concerned product is not  individually manufactured or custom-made in a non-series process in response to a specific order, and which are installed in a single identified construction work, this information shall be omitted];

9.	The performance of the product identified in points 1 and 2 is in conformity with the declared performance in point 8.

This declaration of performance is issued under the sole responsibility of the manufacturer identified in point 4.

Signed for and on behalf of the manufacturer by:

…………………………………………………………………………………………………………

(name and function)

………………..………………	                                               ………………………………

(place and date of issue)	                                                                                         (signature)

NOTE 4:  The number of the declaration of performance shall be assigned by the concerned manufacturer 	(the declaration of performance can be numbered in accordance with the product-type reference number)

NOTE 5:  The number of the notified body is the number assigned by the Commission following the notification procedure.

NOTE 6: For the declaration of properties related to Basic Work Requirement (BWR)  N° 1 “Mechanical resistance and stability” (including such aspects of BWR N°4 “Safety in use”, which relate to mechanical resistance and stability) and aspects of BWR N° 2 “Resistance to fire” in case of structural products, the  methods1, 2, 3a and 3b as appropriate, shall be used  to provide information to be included in the DoP.

The relevant TCs shall complete the Annex ZA included in candidate harmonized standards under development and shall provide for one or several or all methods on the basis of a relevant CEN Guidance document (in preparation).

1.1.4	Clause ZA.3 - CE marking and labelling

(1) 	This clause shall provide details regarding the information associated with the CE marking  set out by Articles 8 and 9 of the CPR.

(2) 	TCs should be aware of the need to permit the use of the “No performance determined” (NPD) option when a characteristic is not regulated for a given end use.

(3) 	In order to ensure that all manufacturers of similar products apply the CE marking in the same way, clause ZA.3 needs to be specific about where the CE marking symbol and the information associated with it is to be placed.

(4) 	According to articles 9.1 and 9.2 of the CPR, the CE marking shall be affixed visibly, legibly and indelibly:

- to the construction product or

- to a label attached to it.

(5) 	Where this is not possible or not warranted on account of the nature of the product:

- it shall be affixed to the packaging or

- to the accompanying documents.

(6) 	The CE marking shall be followed by:

- the two last digits of the year in which it was first affixed,

- the name and the registered address of the manufacturer, or the identifying mark allowing identification of the name and address of the manufacturer easily and without any ambiguity,

- the unique identification code of the product-type, (to be assigned by the manufacturer)

- the reference number of the declaration of performance (to be assigned by the manufacturer)

- the level or class of the performance declared,

- the reference to the harmonised standard  applied,

- the identification number of the notified body, if applicable, and

- the intended use as laid down in the harmonised standard applied.

(7) 	All ZA.3 clauses shall give example(s) of CE marking. If the TC intends to have more than one location (e.g. the CE marking symbol followed by the identification number of the body involved, on the product, with all other information (including the CE marking symbol again) on the accompanying commercial documentation), this shall be stated explicitly.

(8) Where a standard covers more than one type of product and the test/assessment/calculation method depends on the type, the type of product must be indicated.

(9) 	Where the standard provides for more than one method of test/assessment/calculation or of test conditions, and the method or conditions affect the result, the test/assessment/calculation method, and conditions/test configurations used need to be identified together with the stated result in the DoP.

(10) 	In some cases, the unique identification code of the product-type may include some factual information (e.g. dimensions) where a TC wishes, but it cannot contain non-harmonized product performance information.

(11) 	For products intended to be marketed within the EU, information regarding the essential characteristics may not be given without affixing the CE marking on the product. This means, for example, that if such information is given on the product, the CE marking symbol is also required to appear near to it on the product.

(12) 	Part or all of the CE marking information may be given in the form of a designation or symbols, but only if such a designation system or symbols are defined in the standard. When using a designation, care has to be taken to ensure that only information concerning essential characteristics is included in it.

2	Template for Annex ZA

Annex ZA
(informative)

Clauses of this European Standard addressing the provisions of the EU Construction Products Regulation

ZA.1	Scope and relevant characteristics

This European Standard has been prepared under Mandate M [insert here the number and the title of the mandate] given to CEN by the European Commission and the European Free Trade Association.

If this European standard is cited in the Official Journal of the European Union (OJEU), the clauses of this standard, shown in this annex, are considered  to meet the provisions of the relevant mandate, under the Regulation (EU) No. 305/2011.

This annex deals with the CE marking of the [insert here the name of the construction products as given in the title of the standard] intended for the uses indicated in Table(s) ZA.1.1 to ZA.1.n and shows the relevant clauses applicable.

This annex has the same scope as in Clause 1 of this standard related to the aspects covered by the mandate and is defined by Table(s) ZA.1.1 to ZA.1.n.

Table ZA.1.1 — Relevant clauses for product [A] and intended use [1]

| Product: [insert here the name of product A as given in the scope of the standard] Intended use: [insert here the intended use 1] |  |  |  |
| --- | --- | --- | --- |
| Essential Characteristics [1] | Clauses in this and other European Standard(s) related to essential characteristics [2] | Regulatory classes [3] | Notes [4] |
| [name of essential characteristic 1 as given in the accepted answer to the mandate] |  |  |  |
| [name of essential characteristic 2 as given in the accepted answer to the mandate] |  |  |  |
| [name of EC n as given in the accepted answer to the mandate] |  |  |  |
| [durability of essential characteristic 1] |  |  |  |
| [durability of essential characteristic n] |  |  |  |

Table ZA.1.2 — Relevant clauses for product [A] and intended use [2]

| Product: [insert here the name of product A as given in the scope of the standard] Intended use: [insert here the intended use 2] |  |  |  |
| --- | --- | --- | --- |
| Essential Characteristics [1] | Clauses in this and other European Standard(s) related to essential characteristics [2] | Regulatory classes [3] | Notes [4] |
| [name of essential characteristic 1 as given in the accepted answer to the mandate] |  |  |  |
| [name of essential characteristic 2 as given in the accepted answer to the mandate] |  |  |  |
| [name of essential characteristic n as given in the accepted answer to the mandate] |  |  |  |
| [durability of essential characteristic 1] |  |  |  |
| [durability of essential characteristic n] |  |  |  |

Table ZA.1.3 — Relevant clauses for Product [B] and intended use [1]

| Product: [insert here the name of product B as given in the scope of the standard] Intended use: [insert here the intended use 1] |  |  |  |
| --- | --- | --- | --- |
| Essential Characteristics [1] | Clauses in this and other European Standard(s) related to essential characteristics [2] | Regulatory classes [3] | Notes [4] |
| [name of essential characteristic 1 as given in the accepted answer to the mandate] |  |  |  |
| [name of essential characteristic 2 as given in the accepted answer to the mandate] |  |  |  |
| [name of essential characteristic n as given in the accepted answer to the mandate] |  |  |  |
| [durability of essential characteristic 1] |  |  |  |
| [durability of essential characteristic n] |  |  |  |

Table ZA.1.n — Relevant clauses for Product [n] and intended use [n]

| Product: [insert here the name of product n as given in the scope of the standard] Intended use: [insert here the intended use n] |  |  |  |
| --- | --- | --- | --- |
| Essential Characteristics [1] | Clauses in this and other European Standard(s) related to essential characteristics [2] | Regulatory classes [3] | Notes [4] |
| [name of essential characteristic 1 as given in the accepted answer to the mandate] |  |  |  |
| [name of essential characteristic 2 as given in the accepted answer to the mandate] |  |  |  |
| [name of essential characteristic n as given in the accepted answer to the mandate] |  |  |  |
| [durability of essential characteristic 1] |  |  |  |
| [durability of essential characteristic n] |  |  |  |

The declaration of the product performance related to certain essential characteristics is not required in those Member States (MS) where there are no regulatory requirements on these essential characteristics for the intended use of the product.

In this case, manufacturers placing their products on the market of these MS are not obliged to determine nor declare the performance of their products with regard to these essential characteristics and the option “No performance determined” (NPD) in the information accompanying the CE marking and in the declaration of performance (see ZA.3) may be used for those essential characteristics.

[NOTE 1: Each of the essential characteristics for the product and the intended use, should be listed here, including their durability as foreseen in the accepted answer to the mandate.

NOTE 2: The provisions under Clause 4 should be referred to here. Clauses in which reference to the determination method is given are recommended, although where these methods are not included in the clause, they can be referred to under "Notes" in the table. See NOTE 4.

NOTE 3: Threshold levels and  classes of performance, if any, referred to under these requirement clauses in the standard, if any, are not to be listed here except if they are recognized as regulatory classes and levels of performance.

NOTE 4: In the column "Notes", the method of expressing the characteristic should be mentioned i.e. units, accuracy, steps. This column may also indicate the test/assessment/calculation method and refer to threshold  levels or (technical)  classes of performance, where these have been defined in the standard.]

ZA.2	Procedure for AVCP of [insert here the name of construction product as given in the scope of the standard]

ZA.2.1	System(s) of AVCP

The AVCP system(s) of [insert here the name of the construction products as given in the scope of the standard] indicated in Table(s) ZA.1.1 to ZA.1.n, established by EC Decision(s) [insert here the number of the EC Decision(s)]  [(insert here the No. of OJEU)] is shown in Table ZA.2 for the indicated intended use(s) and relevant level(s) or class(es) of performance.

Table ZA.2 — System(s) of AVCP

| Product(s) [5] | Intended use(s) [6] | Level(s) or class(es) of performance [7] | AVCP system(s) [8] |
| --- | --- | --- | --- |
| [insert here the name of the construction products as given in the EC Decision or delegated acts] | [insert here the intended use of the construction products as given in the EC Decision or delegated acts] | [insert here the level(s) or class(es) of performance as given in the EC Decision or delegated acts] | [insert here the AVCP system as given in the title of the EC Decision or delegated acts] |
| System 1+: See Regulation (EU) No. 305/2011 (CPR) Annex V, 1.1 with audit testing of samples taken before placing the product on the market. System 1: See Regulation (EU) No. 305/2011 (CPR) Annex V, 1.2 System 2+: See Regulation (EU) No. 305/2011 (CPR) Annex V, 1.3 including certification of the factory production control by a notified production control certification body on the basis of initial inspection of the manufacturing plant and of factory production control as well as of continuous surveillance, assessment and evaluation of factory production control. System 3: See Regulation (EU) No. 305/2011 (CPR) Annex V, 1.4 System 4: See Regulation (EU) No. 305/2011 (CPR) Annex V, 1.5 [10] |  |  |  |

[NOTE 5: The name of the products shall appear as in the EC Decision(s) or delegated act. All products not covered by the scope of Annex ZA can be omitted. However, where the reference to products included in the above-mentioned acts is too generic, TCs may complement the wording used for making specific reference to product(s). If this is the case, it shall be done outside the Table extracted from the Commission acts.

NOTE 6: The intended use shall appear as in EC Decision(s) or delegated act. All intended uses not covered by the scope of Annex ZA may be omitted.

NOTE 7: The level(s) or class(es) of performance shall appear as in EC Decision(s) or delegated act. All level(s) or class(es)  of performance not covered by the scope of Annex ZA may be omitted.

NOTE 8: The AVCP systems for the products covered shall appear as in EC Decision(s) or delegated act.

NOTE 9: Any reference to non-relevant AVCP system(s) shall be removed.]

The AVCP of the [insert here the name of the construction products as given in the scope of the standard] in Table(s) ZA.1.1 to ZA.1.n shall be according to the AVCP procedures indicated in Table(s) ZA.3.1 to ZA.3.n resulting from application of the clauses of this or other European Standard indicated therein. The content of tasks of the notified body shall be limited to those essential characteristics as provided for, if any, in Annex III of the relevant mandate and to those that the manufacturer intends to declare.

Table ZA.3.1 — Assignment of AVCP tasks for [insert here the name of the construction products as given in the scope of the standard] under system 1+ [10]

| Tasks |  | Content of the task [11] | AVCP clauses to apply [12] |
| --- | --- | --- | --- |
| Tasks for the manufacturer | Factory production control (FPC) | Parameters related to essential characteristic of Table ZA.1 relevant for the intended use which are declared |  |
|  | Further testing of samples taken at factory according to the prescribed test plan | Essential characteristic of Table ZA.1 relevant for the intended use which are declared |  |
| Tasks for the notified product certification body | determination of the product-type on the basis of type testing (including sampling), type calculation, tabulated values or descriptive documentation of the product | [Essential characteristic of Table ZA.1 relevant for the intended use indicated in Annex III of the mandate] |  |
|  | Initial inspection of the manufacturing plant and of FPC | Parameters related to essential characteristic of Table ZA.1, relevant for the intended use which are declared, namely [those indicated in Annex III of the mandate]. Documentation of the FPC. |  |
|  | Continuous surveillance, assessment and evaluation of FPC | Parameters related to essential characteristic of Table ZA.1, relevant for the intended use which are declared, namely [those indicated in Annex III of the mandate] Documentation of the FPC. |  |
|  | Audit testing of samples taken before placing the product on the market | [Essential characteristic of Table ZA.1 relevant for the intended use which are declared], namely [those indicated in Annex III of the mandate] |  |

Table ZA.3.2 — Assignment of AVCP tasks for [insert here the name of the construction products as given in the scope of the standard] under system 1 [10]

| Tasks |  | Content of the task [11] | AVCP clauses to apply [12] |
| --- | --- | --- | --- |
| Tasks for the manufacturer | Factory production control (FPC) | Parameters related to essential characteristics of Table ZA.1 relevant for the intended use which are declared |  |
|  | Further testing of samples taken at factory according to the prescribed test plan | Essential characteristics of Table ZA.1 relevant for the intended use which are declared |  |
| Tasks for the notified product certification body | determination of the product type on the basis of type testing (including sampling), type calculation, tabulated values or descriptive documentation of the product | [Essential characteristics of Table ZA.1 relevant for the intended use indicated in Annex III of the mandate] |  |
|  | Initial inspection of manufacturing plant and of FPC | Parameters related to essential characteristics of Table ZA.1, relevant for the intended use which are declared, namely [those indicated in Annex III of the mandate]. Documentation of the FPC. |  |
|  | Continuous surveillance, assessment and evaluation of FPC | Parameters related to essential characteristics of Table ZA.1, relevant for the intended use which are declared, namely [those indicated in Annex III of the mandate]. Documentation of FPC |  |

Table ZA.3.3 — Assignment of AVCP tasks for [insert here the name of the construction products as given in the scope of the standard] under system 2+

| Tasks |  | Content of the task [11] | AVCP clauses to apply [12] |
| --- | --- | --- | --- |
| Tasks for the manufacturer | Factory production control (FPC) | Parameters related to essential characteristics of Table ZA.1 relevant for the intended use which are declared |  |
|  | determination of the product-type on the basis of type testing (including sampling), type calculation, tabulated values or descriptive documentation of the product | Parameters related to essential characteristics of Table ZA.1 relevant for the intended use which are declared |  |
|  | Further testing of samples taken at factory according to the prescribed test plan | Essential characteristics of Table ZA.1 relevant for the intended use which are declared |  |
| Tasks for the notified production control certification body | Initial inspection of the manufacturing plant and of FPC | Parameters related to essential characteristics of Table ZA.1, relevant for the intended use which are declared, namely [those indicated in Annex III of the mandate]. Documentation of the FPC. |  |
|  | Continuous surveillance, assessment and evaluation of FPC | Parameters related to essential characteristics of Table ZA.1, relevant for the intended use which are declared, namely [those indicated in Annex III of the mandate]. Documentation of the FPC. |  |

Table ZA.3.4 — Assignment of AVCP tasks for [insert here the name of the construction products as given in the scope of the standard] under system 3

| Tasks |  | Content of the task [11] | AVCP clauses to apply [12] |
| --- | --- | --- | --- |
| Tasks for the manufacturer | Factory production control (FPC) | Parameters related to essential characteristics of Table ZA.1 relevant for the intended use which are declared |  |
| Tasks for a notified testing laboratory | Determination of the product-type on the basis of type testing (based on sampling carried out by the manufacturer), type calculation, tabulated values or descriptive documentation of the product | [essential characteristics of Table ZA.1 relevant for the intended use indicated in Annex III of the mandate] |  |

Table ZA.3.5 — Assignment of AVCP tasks for [insert here the name of the construction products as given in the scope of the standard] under system 4

| Tasks |  | Content of the task [11] | AVCP clauses to apply [12] |
| --- | --- | --- | --- |
| Tasks for the manufacturer | Factory production control (FPC) | Parameters related to essential characteristics of Table ZA.1 relevant for the intended use |  |
|  | Determination of the product-type on the basis of type testing, type calculation, tabulated values or descriptive documentation of the product | Essential characteristics of Table ZA.1 relevant for the intended use which are declared |  |

[NOTE 10: for products having more than one intended use specified in the following families, the tasks for the notified body, related to the applicable systems of AVCP, are cumulative.

NOTE 11: The text within inverted commas and in italics shall be replaced by the actual list of relevant characteristics or parameters established for the relevant limited task in 3.1, 3.2, 3.3 or 3.4 of Annex III of the mandate.  When the product has more than one intended use it is recommended, where practicable, that the assignment of tasks is brought together in one or more of the tables above. “relevant” in this column relates to the fact that some characteristics of Table ZA.1 may not be assessed, depending on the intended use of the product.

NOTE 12: The relevant sub-clauses of the clause on AVCP (Clause 6 of the standard) should be referred to herein. Reference to specific parts of the tables likely to be in these sub-clauses may be made if considered necessary for the correct application of the sub-clause.]

ZA.2.2	Declaration of performance (DoP)

ZA.2.2.1 	General

The manufacturer draws up the DoP and affixes the CE marking on the basis of the different AVCP systems set out in Annex V of the Regulation (EU) No 305/2011:

In case of products under system 1+

-	the factory production control and the further testing of samples taken at the factory according to the prescribed test plan, carried out by the manufacturer; and

-	the certificate of constancy of performance issued by the notified product certification body on the basis of determination of the product type on the basis of type testing (including sampling), type calculation, tabulated values or descriptive documentation of the product; initial inspection of the manufacturing plant and of factory production control; continuous surveillance, assessment and evaluation of factory production control and audit-testing of samples taken before placing the product on the market.

In case of products under system 1

-	the factory production control and further testing of samples taken at the factory according to the prescribed test plan, carried out by the manufacturer; and

-	the certificate of constancy of performance issued by the notified product certification body on the basis of determination of the product type on the basis of type testing (including sampling), type calculation, tabulated values or descriptive documentation of the product; initial inspection of the manufacturing plant and of factory production control and continuous surveillance, assessment and evaluation of factory production control.

In case of products under system 2+

-	the determination of the product-type on the basis of type testing (including sampling), type calculation, tabulated values or descriptive documentation of the product;  the factory production control and the testing of samples taken at the factory according to the prescribed test plan, carried out by the manufacturer; and

-	the certificate of conformity of the factory production control, issued by the notified production control certification body on the basis of:

-	initial inspection of the manufacturing plant and of factory production control and

-	 continuous surveillance, assessment and evaluation of factory production control.

In case of products under system 3

-	the factory production control carried out by the manufacturer; and

-	the determination of the product-type on the basis of type testing (based on sampling carried out by the manufacturer), type calculation, tabulated values or descriptive documentation of the product, carried out by the notified testing laboratory.

In case of products under system 4

-	the factory production control carried out by the manufacturer

-	the determination by the manufacturer of the product-type on the basis of type testing, type calculation, tabulated values or descriptive documentation of the product.

ZA.2.2.2	Content

The model of the DoP is provided in Annex III of the Regulation (EU) No 305/2011.

According to this Regulation, the DoP shall contain, in particular, the following information:

-	the reference of the product-type for which the declaration of performance has been drawn up;

-	the AVCP system or systems of the construction product, as set out in Annex V of the CPR;

-	the reference number and date of issue of the harmonised standard which has been used for the assessment of each essential characteristic;

-	where applicable, the reference number of the Specific Technical Documentation used and the requirements with which the manufacturer claims the product complies.

The DoP shall in addition contain:

(a)	the intended use or uses for the construction product, in accordance with the applicable harmonised technical specification;

(b)	the list of essential characteristics, as determined in the harmonised technical specification for the declared intended use or uses;

(c)	the performance of at least one of the essential characteristics of the construction product, relevant for the declared intended use or uses;

(d)	where applicable, the performance of the construction product, by levels or classes, or in a description, if necessary based on a calculation in relation to its essential characteristics determined in accordance with the Commission determination regarding those essential characteristics for which the manufacturer shall declare the performance of the product when it is placed on the market or the Commission determination regarding threshold levels for the performance in relation to the essential characteristics to be declared.

(e)	the performance of those essential characteristics of the construction product which are related to the intended use or uses, taking into consideration the provisions in relation to the intended use or uses where the manufacturer intends the product to be made available on the market;

(f)	for the listed essential characteristics for which no performance is declared, the letters "NPD" (No Performance Determined);

Regarding the supply of the DoP, article 7 of the Regulation (EU) No 305/2011 applies.

The information referred to in Article 31 or, as the case may be, in Article 33 of Regulation (EC) No 1907/2006, (REACH) shall be provided together with the DoP.

[In case of structural products, methods for determining the properties related to  Basic Work Requirement (BWR)  N° 1 “Mechanical resistance and stability” (including such aspects of BWR N°4 “Safety in use”, which relate to mechanical resistance and stability) and aspects of BWR N° 2 “Resistance to fire”, to be declared  as information accompanying the CE marking.

The relevant TCs shall complete the Annex ZA included in candidate harmonized standards under development and shall provide for one or several or all methods on the basis of relevant CEN Guidance document]

ZA.2.2.3	Example of DoP

The following gives an example of a filled-in DoP for vertical air/flue terminals

DECLARATION OF PERFORMANCE

No. 001CPR2013-07-14

- Unique identification code of the product-type:

Positive pressure air/flue terminal with metal flue duct for C62- and C63-type gas appliances

T120- P1- D-Vm-L40045- O50

2	Type, batch or serial number or any other element allowing identification of the construction product as required under Article 11(4):

Positive pressure air/flue terminal with metal flue duct for C62- and C63-type gas appliances

T120- P1- D-Vm-L40045- O50

3.	Intended use or uses of the construction product, in accordance with the applicable harmonised technical specification, as foreseen by the manufacturer:

Convey air for combustion, and the products of combustion from appliances to the outside atmosphere.

4.	Name, registered trade name or registered trade mark and contact address of the manufacturer as required under Article 11(5):

AnyCo SA,

PO Box 21

B-1050 Brussels, Belgium

Tel. +32987654321

Fax: +32123456789

Email: anyco.sa@provider.be

5.	Where applicable, name and contact address of the authorised representative whose mandate covers the tasks specified in Article 12(2):

Anyone Ltd

Flower Str. 24

West Hamfordshire

UK-589645 United Kingdom

Tel. +44987654321

Fax: +44123456789

e-mail: anyone.ltd@provider.uk

6.	System or systems of assessment and verification of constancy of performance of the construction product as set out in CPR, Annex V:

System 2+

7.	In case of the declaration of performance concerning a construction product covered by a harmonised standard:

Notified factory production control certification body No. 5678 performed  the initial inspection of the manufacturing plant and of factory production control and the continuous surveillance, assessment and evaluation of factory production control and issued the certificate of conformity of the factory production control.

8.	Declared performance

| Essential characteristics | Performance | Harmonised technical specification |
| --- | --- | --- |
| Compressive strength | Pass | EN 14989-1: 2007 |
| Resistance to fire | 050 |  |
| Gas tightness/leakage: - of the flue - of the air supply duct | ≤ 0,006 l s-1 m-2 (under a positive pressure of 200 Pa) ≤ 0,28 l s-1 m-2 (under a positive pressure of 40 Pa) |  |
| Flow resistance coefficient: - of the flue - of the air supply duct | 1,5 (declared) 2,5 (declared) |  |
| Thermal resistance of air/flue terminal: - with separate air/flue configuration - with concentric air/flue configuration | 0,5 m2K/W (declared) 0,35 m2K/W (declared) |  |
| Thermal shock | Pass |  |
| Flexural tensile strength | Pass |  |
| Durability: - against chemicals - against corrosion - freeze thaw | Pass Pass Pass |  |

10.	The performance of the product identified in points 1 and 2 is in conformity with the declared performance in point 8.

This declaration of performance is issued under the sole responsibility of the manufacturer identified in point 4.

Signed for and on behalf of the manufacturer by:

………………………………………………………………………………………………………………………………

(name and function)

…………………………………..……………	                              …………………………………..……………

(place and date of issue)	                                                                                                        (signature)

ZA.3	CE marking and labelling

The CE marking symbol shall be in accordance with the general principles set out in Article 30 of Regulation (EC) No 765/2008 and shall be affixed visibly, legibly and indelibly [see clause 1.1.2.1.3]:

- 	to the [insert here the name of the construction products as given in the title of the standard]

or

- 	to a label attached to it.

Where this is not possible or not warranted on account of the nature of the product, it shall be affixed:

- 	to the packaging

or

-	to the accompanying documents.

NOTE: in addition to the above, clause ZA.3 of Annex ZA of the standard could include provisions to be followed where it is intended to split the information accompanying the CE marking and to place them in different locations

The CE marking shall be followed by:

-	the last two digits of the year in which it was first affixed,

  - the name and the registered address of the manufacturer, or the identifying mark allowing identification of the name and address of the manufacturer easily and without any ambiguity,

  - the unique identification code of the product-type

-	the reference number of the declaration of performance [see example of DoP]

-	the level or class of the performance declared [13 see examples of the declaration in annex]

-	the dated reference to the harmonised technical specification applied [14]

  - the identification number of the notified body, [only for products under systems 1+, 1, 2+ and 3],

  - the intended use as laid down in the harmonised technical specification applied.

The CE marking shall be affixed before the construction product is placed on the market. It may be followed by a pictogram or any other mark notably indicating a special risk or use.

NOTE 13:  Reference shall be made to the list of essential characteristics as they appears in Table ZA.1 of the standard, which shall be included in the table appearing in the declaration of performance

NOTE 14:  In case of harmonized standards, the reference to be used shall be the same appeared in the OJEU.

Examples:

  - EN 450-1:2005+A1:2007

-	EN 671-1:2001; EN 671-1:2001/AC:2002

-	EN 1155:1997; EN 1155:1997/A1:2002; EN 1155:1997/A1:2002/AC:2006

- EN 1457:1999; EN 1457:1999/A1:2002; EN 1457:1999/AC:2006; EN 1457:1999/A1:2002/AC:2007

-	EN 13241-1:2003; EN 13242:2002+A1:2007

In case of Specific Technical Documentation (STD), the reference to be used shall be the number assigned to the STD by the manufacturer

Figures ZA.1 to ZA.5 give examples of the information related to products subject to AVCP under each of the different systems to be given on the [to be specified as decided in clause ZA.3] [see clause 1.1.4].

| 0123 |  | CE marking, consisting of the “CE”-symbol Identification number of the product certification body |
| --- | --- | --- |
| AnyCo Ltd, PO Box 21, B-1050, Brussels, Belgium 13 00001-CPR-2013/05/12 |  | name and the registered address of the manufacturer, or identifying mark Last two digits of the year in which the marking was first affixed reference number of the DoP |
| EN 123 - 5: 2009 Product A intended to be used in (e.g. curtain walling, fire compartmentation, etc.) essential characteristic 1: 50N/cm2 essential characteristic 2: Pass essential characteristic 3: Class A1 essential characteristic 4: RE 60 essential characteristic n: xxx Durability of essential characteristic 1: expressed as indicated in the DoP Durability of essential characteristic n: expressed as indicated in the DoP Dangerous substance X : Less than 0,2 ppm |  | No. of European standard applied, as referenced in OJEU (see note 14) Unique identification code of the product-type Intended use of the product as laid down in the European standard applied Level or class of the performance declared [see note 15] |

Figure ZA.1 — Example CE marking information of products under AVCP system 1+  [to be specified as decided in ZA.3] [see clause 1.1.2.1.3].

NOTE 15: An informative database of European and national provisions on dangerous substances is available at the Construction web site on EUROPA accessed through: http://ec.europa.eu/enterprise/construction/cpd-ds/.

| 0123 |  | CE marking, consisting of the “CE”-symbol Identification number of the product certification body |
| --- | --- | --- |
| AnyCo Ltd, PO Box 21, B-1050, Brussels, Belgium 13 00001-CPR-2013/05/12 |  | name and the registered address of the manufacturer, or identifying mark Last two digits of the year in which the marking was first affixed Reference number of the DoP |
| EN 123 - 5: 2009 Product A intended to be used in (e.g. curtain walling, fire compartmentation, etc.) essential characteristic 1: 50N/cm2 essential characteristic 2: Pass essential characteristic 3: Class A1 essential characteristic 4: RE 60 essential characteristic n: xxx Durability of essential characteristic 1: expressed as indicated in the DoP Durability of essential characteristic n: expressed as indicated in the DoP Dangerous substance X : Less than 0,2 ppm |  | No. of European standard applied, as referenced in OJEU (see note 14) Unique identification code of the product-type Intended use of the product as laid down in the European standard applied Level or class of the performance declared [see note 15] |

Figure ZA.2 — Example CE marking information of products under AVCP system 1 [to be specified as decided in ZA.3] [see clause 1.1.2.1.3].

| 4567 |  | CE marking, consisting of the “CE”-symbol Identification number of the notified production control certification body |
| --- | --- | --- |
| AnyCo Ltd, PO Box 21, B-1050, Brussels, Belgium 13 00001-CPR-2013/05/12 |  | name and the registered address of the manufacturer, or identifying mark Last two digits of the year in which the marking was first affixed Reference number of the DoP |
| EN 123 - 5: 2009 Product A intended to be used in (e.g. curtain walling, fire compartmentation, etc.) essential characteristic 1: 50N/cm2 essential characteristic 2: Pass essential characteristic 3: Class A1 essential characteristic 4: RE 60 essential characteristic n: xxx Durability of essential characteristic 1: expressed as indicated in the DoP Durability of essential characteristic n: expressed as indicated in the DoP Dangerous substance X : Less than 0,2 ppm |  | No. of European standard applied, as referenced in OJEU (see note 14) Unique identification code of the product-type Intended use of the product as laid down in the European standard applied Level or class of the performance declared [see note 15] |

Figure ZA.3 — Example CE marking information of products under AVCP system 2+ [to be specified as decided in ZA.3] [see clause 1.1.2.1.3].

| 8910 |  | CE marking, consisting of the “CE”-symbol Identification number of the notified test laboratory |
| --- | --- | --- |
| AnyCo Ltd, PO Box 21, B-1050, Brussels, Belgium 13 00001-CPR-2013/05/12 |  | name and the registered address of the manufacturer, or identifying mark Last two digits of the year in which the marking was first affixed Reference number of the DoP |
| EN 123 - 5: 2009 Product A intended to be used in (e.g. curtain walling, fire compartmentation, etc.) essential characteristic 1: 50N/cm2 essential characteristic 2: Pass essential characteristic 3: Class A1 essential characteristic 4: RE 60 essential characteristic n: xxx Durability of essential characteristic 1: expressed as indicated in the DoP Durability of essential characteristic n: expressed as indicated in the DoP Dangerous substance X : Less than 0,2 ppm |  | No. of European standard applied, as referenced in OJEU (see note 14) Unique identification code of the product-type Intended use of the product as laid down in the European standard applied Level or class of the performance declared [see note 15] |

Figure ZA.4 — Example CE marking information of products under AVCP system 3 [to be specified as decided in ZA.3] [see clause 1.1.2.1.3].

|  |  | CE marking, consisting of the “CE”-symbol |
| --- | --- | --- |
| AnyCo Ltd, PO Box 21, B-1050, Brussels, Belgium 13 00001-CPR-2013/05/12 |  | name and the registered address of the manufacturer, or identifying mark Last two digits of the year in which the marking was first affixed Reference number of the DoP |
| EN 123 - 5: 2009 Product A intended to be used in (e.g. curtain walling, fire compartmentation, etc.) essential characteristic 1: 50N/cm2 essential characteristic 2: Pass essential characteristic 3: Class A1 essential characteristic 4: RE 60 essential characteristic n: xxx Durability of essential characteristic 1: expressed as indicated in the DoP Durability of essential characteristic n: expressed as indicated in the DoP Dangerous substance X : Less than 0,2 ppm |  | No. of European standard applied, as referenced in OJEU (see note 14) Unique identification code of the product-type Intended use of the product as laid down in the European standard applied Level or class of the performance declared [see note 15] |

Figure ZA.5 — Example CE marking information of products under AVCP system 4 [to be specified as decided in ZA.3] [see clause 1.1.2.1.3].

Regulation (EU) No 305/2011 of the European Parliament and of the Council  of 9 March 2011 laying down harmonised conditions for the marketing of construction products and repealing Council Directive 89/106/EEC. (OJEU L88 of 4.4.2011) (to be known as the Construction Product Regulation – CPR)